## Project Report

1. Title of the Project Physics-Informed Machine Learning for Hydrodynamic Coefficient Prediction of Square Cylinders

2. Field / Area of Project Fluid Dynamics, Machine Learning, Data Science, and Software Engineering

3. Prior Publications and Patents on the Project

| Sr. No. | Publication/Patent | Description | Relevance to Project Findings |
| --- | --- | --- | --- |
| 1 | West & Apelt (1982) | Blockage correction formula for experimental data | Used to correct experimental CD values in the preprocessing module |
| 2 | Ota et al. (1994) | Blockage and Strouhal number correction | Integrated into the 3-formula averaging method for data cleaning |
| 3 | Chen et al. (2020) | XGBoost for hydrodynamic prediction | Provided the foundational ML framework for predicting flow coefficients |
| 4 | Lundberg & Lee (2017) | SHAP explainability | Basis for the explainability module to interpret model decisions |

4. Summary and Background of the Project

- a) Summary of the Project: This project implements a production-ready machine learning system using XGBoost to predict hydrodynamic coefficients (CD, CL, St) for flow past square cylinders. It features a modular architecture that integrates flow physics, bi-stable flow handling, and SHAP-based explainability.

- b) Background of the Project: Predicting forces on square cylinders is vital for structural engineering (buildings, bridges). Traditional methods like experimental testing or numerical simulations (DNS/LES) are either expensive or computationally intensive.

- c) Addressing the Gap / Novelty: The project addresses the lack of fast, interpretable prediction tools by fusing domain expertise (blockage corrections, regime classification) with ML. It uniquely handles the "bi-stable" nature of flow—where multiple states exist for the same conditions—using specialized pre- and post-processing branches.

5. Objectives of the Project

- Data Integration: Fuse experimental and numerical data with physics-guided corrections.

- Model Development: Train separate, optimized XGBoost models for each hydrodynamic coefficient (CD1, CD2, CL, St).

- Explainability: Utilize SHAP analysis to provide transparent insights into how the models make predictions.

- Automation: Create a CLI-based pipeline for seamless data processing, training, and reporting.

6. Working Principle of the Project (in brief)

The system operates as a physics-informed data pipeline:

- Capture/Generate: Loads literature data or generates synthetic samples with realistic physics.

- Process: Applies blockage corrections (West & Apelt, Ota, Glauert) and filters data by turbulence intensity and mesh quality.

- Recognize/Classify: Automatically identifies flow regimes (Extended-body, Reattachment, Co-shedding) and detects drag crisis regions.

- Analyze: Performs 5-fold cross-validation and grid search to find optimal hyperparameters.

- Execute: Trains XGBoost models and handles bi-stable branches for the Strouhal number (St).

- Feedback: Generates comprehensive visualizations, SHAP plots, and an HTML report for validation.

7. Description of the Project in Detail

The system is composed of specialized Python modules:

- preprocessing.py: Handles the 3-formula blockage correction and feature engineering (e.g., log10Re, 1/LD).

- flow_physics.py: Encapsulates domain knowledge for regime classification and drag crisis detection.

- models.py: Manages the XGBoost training logic and 5-fold cross-validation.

- bistable.py: Manages dual-branch models for bi-stable flow regions.

- shap_explainer.py: Computes SHAP values to visualize feature importance and dependence.

- visualizer.py: Generates 15+ plot types, including regime maps and residual distributions.

8. Experimental Validation Results

The models achieved high accuracy on holdout test sets:

| Target | R2 | MAE | RMSE |
| --- | --- | --- | --- |
| CD1 (Upstream Drag) | 0.92 | 0.08 | 0.12 |
| CD2(Downstream Drag) | 0.85 | 0.12 | 0.18 |
| CL (RMS Lift) | 0.88 | 0.05 | 0.08 |
| St (Strouhal Number) | 0.90 | 0.008 | 0.012 |

Physics validation confirmed that CD1 correctly predicts the drag crisis (dropping ~30% in the transition zone) and stabilizes in the supercritical plateau.

9. Working Model Link (if any)

https://github.com/Bhavya-2128/XGBOOST.git